108X10W Wall Washer IP

User Manual

# SAFETY INSTRUCTIONS

Please read these instructions carefully. This user guide

contains important information about the installation, usage and maintenance of this fixture.

.    Please keep this Operation Manual for future reference. If unit is sold to another user, make sure they also receive this instruction booklet.

.    Ensure fixture is connected to proper voltage, and that line voltage is not higher than that stated on the fixture.

.    Make sure there are no flammable materials close to the unit while operating.

.    Always disconnect from the power source before servicing or fuse replacement.  Always use the fuse specified in this manual.

.    Always use a safety cable when hanging fixture overhead.

.    Maximum ambient temperature (Ta) is 40°C ( 104° F). Do not operate fixture at temperatures above this rating.

.    In the event of a serious operating problem, stop using the unit immediately.  Repairs must be carried out by trained, authorized personnel. Contact the nearest authorized technical

assistance center. Only OEM spare parts should be used.

.     Do not connect the device to a dimmer pack.

.    Make sure power cord is never crimped or damaged.

.    Never disconnect power cord by pulling or tugging on the cord.

.    Avoid direct eye exposure to the light source during operation.

## Caution! There are no user serviceable parts inside the unit. Do not open the housing or

attempt any repairs yourself. In the unlikely event your unit may require service, please contact your distributor.

Technical Features / Description

.     108 X 10W RGBW Cree LEDs, divided into 12 pixels

.    Variable intensity control 0-100% in 8bit or 16bit control modes

.     Beam spread: 36° standard. Optional beam widths available

.     Refresh rate: 1200HZ

.     Flash Duration control 0-650ms flashes per second

.    Flash Rate control 0-16.7Hz(50Hz)/0-20Hz(60Hz)

.     Continuous blinder/wash effect

.     Flash intensity curve selection

.     User definable fades

.     LCD control panel display with four control buttons

.     RDM available

.    ArtNet available

.     sAcn available

.     IP65 Rated

# SETUP

Disconnect the power cord before replacing the

fuse. Always replace with the correct fuse type.

## FUSE REPLACEMENT

LED WASH BAR 10810 use a 12A 250V slow-blow fuse (5x20mm). To replace fuse:

1.   With a screwdriver turn the fuse cap counter-clockwise to remove fuse cap with fuse.

2.    Replace fuse attached to fuse cap.

3.    Reinsert fuse cap with new fuse and tighten clockwise.

## FIXTURE LINKING

A DMX data link is needed to operate one or more fixtures via a DMX-512 lighting console. The combined number of channels required by all the fixtures on a DMX data link determines the

number of fixtures the DMX data link can support.

### Important: Fixtures on a DMXdata link must be daisy-chained in one single line. To comply

with the EIA-485standard, no more than 32 devices should be connected on one data link.  Connecting more than 32 fixtures on one serial data link without the use of a DMX optically- isolated splitter may result in deterioration of the digital DMX signal.

Maximum recommended DMX data link distance between fixtures: 984 ft. (300 meters). POWER LINKING

The LED WASH BAR 10810 has Neutrik®  PowerCon IN and THRU connections allowing power

linking (daisy- chaining). Depending on the power provided, you should not exceed the power    threshold.  Max. 5 units 10-120V; max. 10 units 208-240V. A maximum 2 units of the LED WASH BAR 10810 can be power linked.

SETTING UP A DMX SERIAL DATA LINK

1)    Connect the maIe 5-pin XLR connector of the data cabIe to the femaIe 5-pin XLR output of the DMX consoIe. Connect the other end of the data cabIe (femaIe 5-pin XLR) to the maIe 5-pin  XLR connector Iocated on the unit.

2)    Connect from the fixture output as stated above to the input of the foIIowing fixture/ and so forth.

3)    Continue Iinking untiI the Iast fixture is connected in your DMX signaI data chain.

## FIXTURE MOUNTING

Orientation

The LED WASH BAR 10810 may be mounted in any position. Each have a yoke with mounting hoIe for cIamps or coupIers. AIways make sure there is adequate room for ventiIation. Do not    obstruct the unit’s fan or vents.

Support Stand

AIways use a professionaI stand rated to support weight greater than the fixture (see technicaI    specifications).  Attach a TVMP spigot to the yoke of the LED WASH BAR 10810 and mount on the stand.

Rigging 一 Always consult a certified rigging specialist before suspending any fixture overhead!

Use ProBurger® coupIers or equivaIent C- or O-type cIamps for attaching to   truss.  It is important never to obstruct the fan or vents pathway.  Adjust the angIe of the fixture by Ioosening both knobs and tiIting the fixture. After

finding the desired position/ retighten both knobs.

.     Always use safety cables!

.    When seIecting instaIIation Iocation/ consider routine maintenance.

.     Never mount fixture where it wiII be exposed to moisture/ high humidity/ extreme temperatures/ or restricted ventiIation.

DMX Channel

9CHMode

| Channel | Value | Function | Description |
| --- | --- | --- | --- |
| 1CH | 0-255 | Master Dimmer | 0-100% |
| 2CH | 0-255 | Fine Dimmer | 16 bit adjustable |
| 3CH | 0-255 | Strobe function |  |
|  | 0-5 | / |  |
|  | 6-55 | Synchronous strobe | From slow to fast |
|  | 56-105 | Pulse strobe | From slow to fast |
|  | 106-155 | Random strobe | From slow to fast |
|  | 156-205 | Fade Strobe1 | From slow to fast Brightness from high to low |
|  | 206-255 | Fade Strobe2 | From slow to fast Brightness from low to high |
| 4CH | 0-255 | RED | 0-100% |
| 5CH | 0-255 | GREEN | 0-100% |
| 6CH | 0-255 | BLUE | 0-100% |
| 7CH | 0-255 | WHITE | 0-100% |
| 8CH | 0-255 | Color Temperature preset | Color temperature 1700K-8000K |
|  | 0-15 | Off light |  |
|  | 16-30 | 8000K | R=255,G=200,B=155,W=255 |
|  | 36-45 | 7500K | R=255,G=174,B=140,W=255 |
|  | 46-60 | 6500K | R=255,G=152,B=124,W=255 |
|  | 61-75 | 6000K | R=255,G=136,B=105,W=255 |
|  | 76-90 | 5600K | R=255,G=157,B=158,W=255 |
|  | 91-105 | 5000K | R=255,G=124,B=090,W=255 |
|  | 106-120 | 4500K | R=255,G=064,B=063,W=255 |
|  | 121-135 | 4000K | R=255,G=032,B=048,W=255 |
|  | 136-150 | 3500K | R=255,G=006,B=033,W=255 |
|  | 151-165 | 3200K | R=255,G=000,B=024,W=255 |
|  | 166-180 | 3000K | R=255,G=000,B=020,W= 162 |
|  | 181-195 | 2700K | R=255,G=000,B=013,W= 113 |
|  | 196-210 | 2500K | R=255,G=000,B=010,W=095 |
|  | 211-225 | 2300K | R=255,G=000,B=007,W=087 |
|  | 226-240 | 2000K | R=255,G=000,B=003,W=068 |
|  | 241-255 | 1700K | R=255,G=000,B=000,W=045 |
| 9CH | 0-255 | Marco Color |  |
|  | 0-45 | No Function |  |
|  | 46-50 | Cyan | R=000,G=255,B=255,W=000 |
|  | 51-55 | Yellow | R=255,G=255,B=000,W=000 |

|  | 56-60 | Magenta | R=255,G=000,B=255,W=000 |
| --- | --- | --- | --- |
|  | 61-65 | Medium Yellow | R=156,G=126,B=000,W=000 |
|  | 66-70 | Straw Tint | R=152,G=115,B=009,W=000 |
|  | 71-75 | Surprise Peach | R=198,G=114,B=009,W=000 |
|  | 76-80 | Fire | R=255,G=056,B=000,W=000 |
|  | 81-85 | Medium Amber | R=255,G=135,B=000,W=000 |
|  | 86-90 | Gold Amber | R=255,G=100,B=000,W=000 |
|  | 91-95 | Dark Amber | R=255,G=065,B=000,W=000 |
|  | 96-100 | Sunrise Red | R=255,G=083,B=002,W=000 |
|  | 101-105 | Light Pink | R=255,G=112,B=000,W=141 |
|  | 106-110 | Medium Pink | R=255,G=087,B=000,W=107 |
|  | 111-115 | Pink Carnation | R=255,G=107,B=000,W=130 |
|  | 116-120 | Light Lavender | R=243,G=117,B=039,W=197 |
|  | 121-125 | Sky Blue | R=000,G=255,B=135,W=000 |
|  | 126-130 | Just Blue | R=000,G=194,B=130,W=000 |
|  | 131-135 | Dark yellow green | R=041,G=219,B=000,W=000 |
|  | 136-140 | Spring Yellow | R=245,G=202,B=000,W=000 |
|  | 141-145 | Light Amber | R=237,G=163,B=000,W=000 |
|  | 146-150 | Straw Tint | R=230,G=160,B=000,W=069 |
|  | 151-155 | Deep Amber | R=255,G=166,B=000,W=000 |
|  | 156-160 | Orange | R=255,G=122,B=000,W=000 |
|  | 161-165 | Light Rose | R=255,G=141,B=031,W=000 |
|  | 166-170 | Middle Rose | R=217,G=130,B=028,W=000 |
|  | 171-175 | Dark Pink | R=255,G=109,B=033,W=000 |
|  | 176-180 | Magenta | R=255,G=020,B=015,W=000 |
|  | 181-185 | Peacock Blue | R=051,G=255,B=051,W=000 |
|  | 186-190 | Steel Blue | R=206,G=255,B=056,W=000 |
|  | 191-195 | Light Blue | R=074,G=255,B=082,W=000 |
|  | 196-200 | Dark Blue | R=000,G=186,B=255,W=000 |
|  | 201-205 | Leaf Green | R=206,G=255,B=000,W=000 |
|  | 206-210 | Dark Green | R=084,G=255,B=013,W=000 |
| 9CH | 211-215 | Mauve | R=227,G=041,B=056,W=000 |
|  | 216-220 | Deep Golden Amber | R=255,G=058,B=000,W=000 |
|  | 221-225 | Pale Lavender | R=255,G=197,B=061,W=000 |
|  | 226-230 | Primary Green | R=077,G=255,B=000,W=000 |
|  | 231-235 | Bright Blue | R=000,G=255,B=087,W=000 |
|  | 235-240 | Apricot | R=255,G=143,B=013,W=000 |
|  | 241-245 | Pale Gold | R=253,G=171,B=026,W=000 |
|  | 246-250 | Deep Orange | R=222,G=084,B=000,W=000 |
|  | 251-255 | Flame Red | R=255,G=046,B=002,W=000 |

| Channel | Value | Function | Description |
| --- | --- | --- | --- |
| 1CH | 0-255 | Master Dimmer | 0-100% |
| 2CH | 0-255 | Fine Dimmer | 16 bit adjustable |
| 3CH | 0-255 | Strobe function |  |
|  | 0-5 | / |  |
|  | 6-55 | Synchronous strobe | From slow to fast |
|  | 56-105 | Pulse strobe | From slow to fast |
|  | 106-155 | Random strobe | From slow to fast |
|  | 156-205 | Fade Strobe1 | From slow to fast Brightness from high to low |
|  | 206-255 | Fade Strobe2 | From slow to fast Brightness from low to high |
| 4CH | 0-255 | RED | 0-100% |
| 5CH | 0-255 | GREEN | 0-100% |
| 6CH | 0-255 | BLUE | 0-100% |
| 7CH | 0-255 | WHITE | 0-100% |
| 8CH | 0-255 | Master Dimmer of Marco Effects | 0-100% |
| 9CH | 0-255 | Fine Dimmer of Marco Effects | 0-100% |
| 10CH | 0-255 | Strobe Function of Marco Effects |  |
|  | 0-5 | / |  |
|  | 6-55 | Synchronous strobe | From slow to fast |
|  | 56-105 | Pulse strobe | From slow to fast |
|  | 106-155 | Random strobe | From slow to fast |
|  | 156-205 | Fade Strobe1 | From slow to fast Brightness from high to low |
|  | 206-255 | Fade Strobe1 | From slow to fast Brightness from low to high |
| 11CH | 0-255 | Marcos RED | Linear 0-100% |
| 12CH | 0-255 | Marcos GREEN | Linear 0-100% |
| 13CH | 0-255 | Marcos BLUE | Linear 0-100% |
| 14CH | 0-255 | Marcos WHITE | Linear 0-100% |
| 15CH | 0-255 | Built-in Program | Program1-Program16 |
|  | 0-5 | / |  |
|  | 6-20 | Program 1 | The color-changes of the build-in effects is controlled by its color CH |
|  | 21-35 | Program 2 | ~ |
|  | 36-50 | Program 3 | ~ |
|  | 51-65 | Program 4 | ~ |
|  | 66-80 | Program 5 | ~ |
|  | 81-95 | Program 6 | ~ |
|  | 96-110 | Program 7 | ~ |

|  | 111-125 | Program 8 | ~ |
| --- | --- | --- | --- |
|  | 126-140 | Program 9 | ~ |
|  | 141-155 | Program 10 | ~ |
|  | 156-170 | Program 11 | ~ |
|  | 171-185 | Program 12 | ~ |
|  | 186-200 | Program 13 | ~ |
|  | 201-215 | Program 14 | ~ |
|  | 216-230 | Program 15 | ~ |
|  | 231-245 | Program 16 | ~ |
|  | 246-255 | Auto-running built- in programs | Program 1-Program 16 Auto-running |
| 16CH | 0-255 | Program Speed | from slow to fast |
| 17CH | 0-255 | Color Temperature preset | Color temperature 1700K-8000K |
|  | 0-15 | Off light |  |
|  | 16-30 | 8000K | R=255,G=200,B=155,W=255 |
|  | 36-45 | 7500K | R=255,G=174,B=140,W=255 |
|  | 46-60 | 6500K | R=255,G=152,B=124,W=255 |
|  | 61-75 | 6000K | R=255,G=136,B=105,W=255 |
|  | 76-90 | 5600K | R=255,G=157,B=158,W=255 |
|  | 91-105 | 5000K | R=255,G=124,B=090,W=255 |
|  | 106-120 | 4500K | R=255,G=064,B=063,W=255 |
|  | 121-135 | 4000K | R=255,G=032,B=048,W=255 |
|  | 136-150 | 3500K | R=255,G=006,B=033,W=255 |
|  | 151-165 | 3200K | R=255,G=000,B=024,W=255 |
|  | 166-180 | 3000K | R=255,G=000,B=020,W=162 |
|  | 181-195 | 2700K | R=255,G=000,B=013,W=113 |
|  | 196-210 | 2500K | R=255,G=000,B=010,W=095 |
|  | 211-225 | 2300K | R=255,G=000,B=007,W=087 |
|  | 226-240 | 2000K | R=255,G=000,B=003,W=068 |
|  | 241-255 | 1700K | R=255,G=000,B=000,W=045 |
| 18CH | 0-255 | Marco Color |  |
|  | 0-45 |  |  |
|  | 46-50 | Cyan | R=000,G=255,B=255,W=000 |
|  | 51-55 | Yellow | R=255,G=255,B=000,W=000 |
|  | 56-60 | Magenta | R=255,G=000,B=255,W=000 |
|  | 61-65 | Medium Yellow | R=156,G=126,B=000,W=000 |
|  | 66-70 | Straw Tint | R=152,G=115,B=009,W=000 |
|  | 71-75 | Surprise Peach | R=198,G=114,B=009,W=000 |
|  | 76-80 | Fire | R=255,G=056,B=000,W=000 |
|  | 81-85 | Medium Amber | R=255,G=135,B=000,W=000 |
|  | 86-90 | Gold Amber | R=255,G=100,B=000,W=000 |
|  | 91-95 | Dark Amber | R=255,G=065,B=000,W=000 |
|  | 96-100 | Sunrise Red | R=255,G=083,B=002,W=000 |

|  | 101-105 | Light Pink | R=255,G=112,B=000,W=141 |
| --- | --- | --- | --- |
|  | 106-110 | Medium Pink | R=255,G=087,B=000,W=107 |
|  | 111-115 | Pink Carnation | R=255,G=107,B=000,W=130 |
|  | 116-120 | Light Lavender | R=243,G=117,B=039,W=197 |
|  | 121-125 | Sky Blue | R=000,G=255,B=135,W=000 |
|  | 126-130 | Just Blue | R=000,G=194,B=130,W=000 |
|  | 131-135 | Dark yellow green | R=041,G=219,B=000,W=000 |
|  | 136-140 | Spring Yellow | R=245,G=202,B=000,W=000 |
|  | 141-145 | Light Amber | R=237,G=163,B=000,W=000 |
|  | 146-150 | Straw Tint | R=230,G=160,B=000,W=069 |
|  | 151-155 | Deep Amber | R=255,G=166,B=000,W=000 |
|  | 156-160 | Orange | R=255,G=122,B=000,W=000 |
|  | 161-165 | Light Rose | R=255,G=141,B=031,W=000 |
|  | 166-170 | Middle Rose | R=217,G=130,B=028,W=000 |
|  | 171-175 | Dark Pink | R=255,G=109,B=033,W=000 |
|  | 176-180 | Magenta | R=255,G=020,B=015,W=000 |
|  | 181-185 | Peacock Blue | R=051,G=255,B=051,W=000 |
|  | 186-190 | Steel Blue | R=206,G=255,B=056,W=000 |
|  | 191-195 | Light Blue | R=074,G=255,B=082,W=000 |
|  | 196-200 | Dark Blue | R=000,G=186,B=255,W=000 |
|  | 201-205 | Leaf Green | R=206,G=255,B=000,W=000 |
|  | 206-210 | Dark Green | R=084,G=255,B=013,W=000 |
|  | 211-215 | Mauve | R=227,G=041,B=056,W=000 |
|  | 216-220 | Deep Golden Amber | R=255,G=058,B=000,W=000 |
|  | 221-225 | Pale Lavender | R=255,G=197,B=061,W=000 |
|  | 226-230 | Primary Green | R=077,G=255,B=000,W=000 |
|  | 231-235 | Bright Blue | R=000,G=255,B=087,W=000 |
|  | 235-240 | Apricot | R=255,G=143,B=013,W=000 |
|  | 241-245 | Pale Gold | R=253,G=171,B=026,W=000 |
|  | 246-250 | Deep Orange | R=222,G=084,B=000,W=000 |
|  | 251-255 | Flame Red | R=255,G=046,B=002,W=000 |
| 19CH | 0-255 | Dimmer Curve |  |
|  | 0-105 | Dimmer Curve 1 |  |
|  | 106-150 | Dimmer Curve 2 |  |
|  | 151-200 | Dimmer Curve 3 |  |
|  | 201-255 | Dimmer Curve 4 | .... |

53CH Mode

| Channel | Value | Function | Description |
| --- | --- | --- | --- |
| 1CH | 0-255 | Master Dimmer | 0-100% |
| 2CH | 0-255 | Fine Dimmer | 16 bit adjustable |
| 3CH | 0-255 | Strobe function |  |
|  | 0-5 | / |  |
|  | 6-55 | Synchronous strobe | From slow to fast |
|  | 56-105 | Pulse strobe | From slow to fast |
|  | 106-155 | Random strobe | From slow to fast |
|  | 156-205 | Fade Strobe1 | From slow to fast Brightness from high to low |
|  | 206-255 | Fade Strobe2 | From slow to fast Brightness from low to high |
| 4CH | 0-255 | Color Temperature preset | Color temperature 1700K-8000K |
|  | 0-15 | Off light |  |
|  | 16-30 | 8000K | R=255,G=200,B=155,W=255 |
|  | 36-45 | 7500K | R=255,G=174,B=140,W=255 |
|  | 46-60 | 6500K | R=255,G=152,B=124,W=255 |
|  | 61-75 | 6000K | R=255,G=136,B=105,W=255 |
|  | 76-90 | 5600K | R=255,G=157,B=158,W=255 |
|  | 91-105 | 5000K | R=255,G=124,B=090,W=255 |
|  | 106-120 | 4500K | R=255,G=064,B=063,W=255 |
|  | 121-135 | 4000K | R=255,G=032,B=048,W=255 |
|  | 136-150 | 3500K | R=255,G=006,B=033,W=255 |
|  | 151-165 | 3200K | R=255,G=000,B=024,W=255 |
|  | 166-180 | 3000K | R=255,G=000,B=020,W=162 |
|  | 181-195 | 2700K | R=255,G=000,B=013,W=113 |
|  | 196-210 | 2500K | R=255,G=000,B=010,W=095 |
|  | 211-225 | 2300K | R=255,G=000,B=007,W=087 |
|  | 226-240 | 2000K | R=255,G=000,B=003,W=068 |
|  | 241-255 | 1700K | R=255,G=000,B=000,W=045 |
| 5CH | 0-255 | Marco Color |  |
|  | 0-45 |  |  |
|  | 46-50 | Cyan | R=000,G=255,B=255,W=000 |
|  | 51-55 | Yellow | R=255,G=255,B=000,W=000 |
|  | 56-60 | Magenta | R=255,G=000,B=255,W=000 |
|  | 61-65 | Medium Yellow | R=156,G=126,B=000,W=000 |
|  | 66-70 | Straw Tint | R=152,G=115,B=009,W=000 |
|  | 71-75 | Surprise Peach | R=198,G=114,B=009,W=000 |
|  | 76-80 | Fire | R=255,G=056,B=000,W=000 |
|  | 81-85 | Medium Amber | R=255,G=135,B=000,W=000 |
|  | 86-90 | Gold Amber | R=255,G=100,B=000,W=000 |
|  | 91-95 | Dark Amber | R=255,G=065,B=000,W=000 |
|  | 96-100 | Sunrise Red | R=255,G=083,B=002,W=000 |

|  | 101-105 | Light Pink | R=255,G=112,B=000,W=141 |
| --- | --- | --- | --- |
|  | 106-110 | Medium Pink | R=255,G=087,B=000,W=107 |
|  | 111-115 | Pink Carnation | R=255,G=107,B=000,W=130 |
|  | 116-120 | Light Lavender | R=243,G=117,B=039,W=197 |
|  | 121-125 | Sky Blue | R=000,G=255,B=135,W=000 |
|  | 126-130 | Just Blue | R=000,G=194,B=130,W=000 |
|  | 131-135 | Dark yellow green | R=041,G=219,B=000,W=000 |
|  | 136-140 | Spring Yellow | R=245,G=202,B=000,W=000 |
|  | 141-145 | Light Amber | R=237,G=163,B=000,W=000 |
|  | 146-150 | Straw Tint | R=230,G=160,B=000,W=069 |
|  | 151-155 | Deep Amber | R=255,G=166,B=000,W=000 |
|  | 156-160 | Orange | R=255,G=122,B=000,W=000 |
|  | 161-165 | Light Rose | R=255,G=141,B=031,W=000 |
|  | 166-170 | Middle Rose | R=217,G=130,B=028,W=000 |
|  | 171-175 | Dark Pink | R=255,G=109,B=033,W=000 |
|  | 176-180 | Magenta | R=255,G=020,B=015,W=000 |
|  | 181-185 | Peacock Blue | R=051,G=255,B=051,W=000 |
|  | 186-190 | Steel Blue | R=206,G=255,B=056,W=000 |
|  | 191-195 | Light Blue | R=074,G=255,B=082,W=000 |
|  | 196-200 | Dark Blue | R=000,G=186,B=255,W=000 |
|  | 201-205 | Leaf Green | R=206,G=255,B=000,W=000 |
|  | 206-210 | Dark Green | R=084,G=255,B=013,W=000 |
|  | 211-215 | Mauve | R=227,G=041,B=056,W=000 |
|  | 216-220 | Deep Golden Amber | R=255,G=058,B=000,W=000 |
|  | 221-225 | Pale Lavender | R=255,G=197,B=061,W=000 |
|  | 226-230 | Primary Green | R=077,G=255,B=000,W=000 |
|  | 231-235 | Bright Blue | R=000,G=255,B=087,W=000 |
|  | 235-240 | Apricot | R=255,G=143,B=013,W=000 |
|  | 241-245 | Pale Gold | R=253,G=171,B=026,W=000 |
|  | 246-250 | Deep Orange | R=222,G=084,B=000,W=000 |
|  | 251-255 | Flame Red | R=255,G=046,B=002,W=000 |
| 6CH | 0-255 | CW1 Red | Dark to light |
| 7CH | 0-255 | CW1 Green | Dark to light |
| 8CH | 0-255 | CW1 Blue | Dark to light |
| 9CH | 0-255 | CW1 White | Dark to light |
| .... | .... | .... | .... |
| .... | .... | .... | .... |
| .... | .... | .... | .... |
| 50CH | 0-255 | CW12 Red | Dark to light |
| 51CH | 0-255 | CW12 Green | Dark to light |
| 52CH | 0-255 | CW12 Blue | Dark to light |
| 53CH | 0-255 | CW12 White | Dark to light |

TECHNICAL SPECIFICATIONS

#### WEIGHT / DIMENSIONS

Length                                                                                                              39.4” (1002mm)    Width                                                                                                                  7.87” (200mm)    Height (w/o yoke)                                                                                               10.7” (271.5mm) Weight                                                                                                               37.5lb.(17 kg)

#### POWER

Operating Voltage                                                                               100-277VAC 50/60Hz Fuse........................................................................................ 12A 250V slow-blow (5x20mm) Power consumption                                                                                                 1400W

#### LIGHT SOURCE

LEDs                                                              108 RGBW Cree LEDs divided into 12 pixels Color                                                                                                                              RGBW

Beam spread                                                                                                                      36°

Intensity control                                                                                                   16bit or 8bit

Refresh rate                                                                                                               From 400 to 24K

#### AVERAGE COLOR WAVELENGTH

Red                                                                                                                              625 nm Green                                                                                                                          530 nm Blue                                                                                                                              460 nm White                                                                                                                           >5700K Refresh rate                                                                                                               1200Hz

#### THERMAL

Max. ambient temperature                                                                        +104°F (+40 °C)

Min. ambient temperature.................................................................................. ..-4 °F (-20 °C) Cooling                                                                                                            air cooled – fan

#### CONTROL / PROGRAMMING

DMX input                                                                              locking 5-pin XLR male socket

DMX output........................................................................... locking 5-pin XLR female socket

DMX pin config                                     pin 1 shield, pin 2 (-), pin 3 (+), pin 4 n/a, pin 5 n/a

Control                                                                       RDM, DMX-512, ArtNet,sAcn, Manual Mode DMX Channels – Basic Mode                                                                                  3,4,6,7,8

DMX Channels - Advanced Mode                                 10,12,13,16,19,20,24,25,32,43,56 Power Connector                                                                               Input: PowerCon 20A

#### IP RATED

IP Rated                                                                                 IP65

Dimension: