User Manual

18*25W Wall Washer IP

SAFETY PRECAUTIONS

This fixture is a sophisticated piece of electronic equipment.To guarantee a smooth operation,it is important to follow all instructions and guidelines in this manual. Professional is not responsible for injuries and/or damages resulting from the misuse of this fixture due to the disregard of the information printed in this manual.Only qualified and/or certified personnel should perform installation of this fixture and only the original rigging parts included with this fixture should be used for installation.Any modifications to the fixture and/ or the included mounting hardware will void the original manufacturer’s warranty and increase the risk of damage and/or personal injury.

PROTECTION CLASS 1 - FIXTURE MUST BE PROPERLY GROUNDED.

THERE ARE NO USER SERVICEABLE PARTS INSIDE THIS UNIT.DO NOT  ATTEMPT ANY REPAIRS YOURSELF,AS DOING SO WILL VOID YOUR  MANUFACTURER’S WARRANTY.DAMAGES RESULTING FROM MODIFICATIONS  TO THIS FIXTURE AND/OR THE DISREGARD OF SAFETY INSTRUCTIONS AND  GUIDELINES IN THIS MANUAL VOID THE MANUFACTURER’S WARRANTY AND  ARE NOT SUBJECT TO ANY WARRANTY CLAIMS AND/OR REPAIRS.

DO NOT PLUG FIXTURE INTO A DIMMER PACK!

NEVER OPEN THIS FIXTURE WHILE IN USE!

UNPLUG POWER BEFORE SERVICING FIXTURE!

NEVER TOUCH FIXTURE DURING OPERATION,AS IT MAY BE HOT! KEEP FLAMMABLE MATERIALS AWAY FROM FIXTURE!

NEVER LOOK DIRECTLY INTO THE LIGHT SOURCE!

RETINA INJURY RISK - MAY INDUCE BLINDNESS!

SENSITIVE PERSONS MAY SUFFER AN EPILEPTIC SHOCK!

ENSURE ALL CONNECTIONS AND END CAPS ARE PROPERLY SEALED WITH  A DIELECTRIC GREASE (AVAILABLE AT MOST ELECTRICAL SUPPLIERS)TO  PREVENT WATER CORROSION AND/OR ELECTRICAL SHORT CIRCUIT.

MINIMUM DISTANCE TO OBJECTS/SURFACES

MUST BE 3.3FEET (1METER)

MAXIMUM TEMP OF EXTERNAL SURFACE 185° F (85°C)

MINIMUM DISTANCE OF FLAMMABLE MATERIALS FROM THE SURFACE IS 1.6  FEET (0 .5 METER)

SAFETY PRECAUTIONS

•     DO NOT TOUCH the fixture housing during operation.Turn OFF the power and allow approximately 15 minutes for the fixture to cool down before servicing.

•     DO NOT shake fixture, and avoid  brute force  when installing  and/or operating  fixture.

•     DO NOT operate fixture if the power cord  is frayed, crimped, damaged and/or  if any of the power cord connectors are damaged and do not insert  into the fixture securely with ease.

•     NEVER force a power cord connector into the fixture.If the power cord or any of  its connectors are damaged, replace it immediately with a new one of the same power rating.

•     DO NOT block any air ventilation slots.

•     All fan and air inlets must remain clean and never blocked.

•     Allow approx.6” (15cm)  between fixture and other devices or a wall for proper cooling.

•     Always disconnect fixture from main power source before performing any type of service and/or cleaning procedure.

•     Only handle the power cord by the plug end.Never pull out the plug by tugging the wire portion of the cord.

•     During the initial operation of this fixture, a light smoke or smell may emit from the  interior of the fixture. This is a normal process and is caused  by excess paint in the  interior of the casing burning off from the heat associated with the lamp and will decrease gradually over time.

•     Consistent operational breaks will ensure fixture will function properly for many years .

•     ONLY use the original packaging and materials to transport the fixture in for service.

MAINTENANCE GUIDELINES

DISCONNECT POWER BEFORE PERFORMING ANY MAINTENANCE!

CLEANING

Frequent cleaning is recommended to ensure proper function,optimized light output, and an extended life.The frequency of cleaning depends on the environment in which the fixture operates: damp,smoky or particularly dirty environments can cause greater accumulation of dirt on the fixture’s optics.Periodically clean the external lens surface with a soft cloth to avoid dirt/debris accumulation.NEVER use alcohol, solvents, or ammonia-based cleaners .

MAINTENANCE

Regular inspections are recommended to ensure proper function and extended life.There are no user serviceable parts inside this fixture.Please refer all other service issues to an authorized our service technician.Should you need any spare parts, please order genuine parts from your local dealer.

Please refer to the following points during routine inspections:

•     A detailed electric check by an approved electrical engineer every three months, to make sure the circuit contacts are in good condition and prevent overheating.

•      Be sure all screws and fasteners are securely tightened at all times.Loose screws may fall out during normal operation, resulting in damage or injury as larger parts could fall.

•     Check for any deformations on the housing, color lenses, rigging hardware,and rigging points (ceiling,suspension, trussing).Deformations in the housing could allow for dust to enter into the fixture.Damaged rigging points or unsecured rigging could cause the fixture to fall and seriously injure a person(s).

•     Electric power supply cables must not show any damage,material fatigue or sediments.NEVER remove the ground prong from the power cable.

OVERVIEW

- FAN

- POWER OUTPUT

- RJ45 OUT

- RJ45 IN

- POWER INPUT

- FAN

- LOCKING SYSTEM

- BRACKET

- DMX OUT

- SYSTEM MENU LCD DISPLAY

- DMX IN

- FUSE

SYSTEM MENU

| Main Menu | Sub Menu | Option/Value | Option/Value |
| --- | --- | --- | --- |
| Address | 001 |  |  |
|  | 512 |  |  |
| Channel Mode | 4CH |  |  |
|  | 10CH |  |  |
|  | 20CH |  |  |
|  | 33CH |  |  |
|  | 37CH |  |  |
|  | 77CH |  |  |
| Setting | Control Mode | DMX |  |
|  |  | ArtNet |  |
|  |  | sACN |  |
|  | Dimmer Mode | Mode 1 |  |
|  |  | Mode 2 |  |
|  |  | Mode 3 |  |
|  |  | Mode 4 |  |
|  | Dimmer Speed | Smooth / Fast |  |
|  | Led Frequency | 800HZ 1200HZ 2000HZ 4000HZ | 6000HZ 8000HZ 16000HZ 24000HZ |
|  | Fan Mode | AUTO |  |
|  |  | Silent |  |
|  |  | FULL |  |
|  | Screen Reverse | NO/YES |  |
|  | LED Remap | NO/YES |  |
|  | Lock Touch | NO/YES |  |
|  | Factory Set | Confirm/Cancel |  |
| Network | Ethernet | IP Mode:CUSTOM |  |
|  |  | IP&Subnet Mask: | 002.000.000.001 255.000.000.000 |
|  | Universe | Net | 0-127 |
|  |  | Sub_Uni | 0-255 |
| Manual | Normal |  |  |
|  | Static | Fixed Color | R |
|  |  |  | G |
|  |  |  | B |
|  |  |  | L |
|  |  |  | GB |
|  |  |  | RB |
|  |  |  | RG |
|  |  |  | RGB |
|  |  |  | RL |
|  |  |  | BL |
|  |  |  | GL |
|  |  |  | RGL |
|  |  |  | RBL |
|  |  |  | GBL |
|  |  |  | RGBL |
|  |  | Manual Color | R |
|  |  |  | G |
|  |  |  | B |
|  |  |  | L |

DMX CHART

## 4CH Mode

| CH | Value | Percent | Function |
| --- | --- | --- | --- |
| 1CH | 0-255 | 0%-100% | Red |
| 2CH | 0-255 | 0%-100% | Green |
| 3CH | 0-255 | 0%-100% | Blue |
| 4CH | 0-255 | 0%-100% | Lime |

10CH Mode

| CH | Value | Percent | Function |
| --- | --- | --- | --- |
| 1CH | 0-255 | 0%-100% | Master Dimmer |
| 2CH | 0-255 | 0%-100% | Fine Dimmer |
| 3CH | 0-255 | 0%-100% | Strobe of Marcos Effect |
|  | 0-5 | 0%-1.9% | No Function |
|  | 6-55 | 2.3%-21.5% | Synchronous strobe |
|  | 56-105 | 21.9%-41.1% | Pulse strobe |
|  | 106-155 | 41.5%-60.7% | Random strobe |
|  | 156-205 | 61.1%-80.3% | Fade Strobe1 |
|  | 206-255 | 80.7%-100% | Fade Strobe2 |
| 4CH | 0-255 | 0%-100% | Red |
| 5CH | 0-255 | 0%-100% | Green |
| 6CH | 0-255 | 0%-100% | Blue |
| 7CH | 0-255 | 0%-100% | Lime |
| 8CH | 0-255 | 0%-100% | Color Temperature preset |
|  | 0-15 | 0%-5.8% | No Function |
|  | 16-30 | 6.2%-11.7% | 8000K（R255,G200, B155, L255) |
|  | 36-45 | 13.7%-20.0% | 7500K(R255,G174, B140, L255) |
|  | 46-60 | 14.1%-23.5% | 6500K(R255,G152, B124, L255) |
|  | 61-75 | 23.9%-29.4% | 6000K(R255,G136, B105, L255) |
|  | 76-90 | 29.8%-35.2% | 5600K（R255,G157, B158, L255) |
|  | 91-105 | 35.6%-41.1% | 5000K(R255,G124, B90, L255) |
|  | 106-120 | 41.5%-47.0% | 4500K(R255,G64, B63, L255) |
|  | 121-135 | 47.4%-52.9% | 4000K(R255,G32, B48, L255) |
|  | 136-150 | 53.3%-58.8% | 3500K(R255,G6, B33, L255) |
|  | 151-165 | 59.2%-64.7% | 3200K(R255,G0, B24, L255) |
|  | 166-180 | 65.0%-70.5% | 3000K(R255,G0, B20, L162) |
|  | 181-195 | 70.9%-76.4% | 2700K(R255,,G0, B13, L113) |
|  | 196-210 | 76.8%-82.3% | 2500K(R255,G0, B10, L95) |
|  | 211-225 | 82.7%-88.2% | 2300K(R255,G0, B7, L87) |
|  | 226-240 | 88.6%-94.1% | 2000K(R255,G0, B3, L68) |
|  | 241-255 | 94.5%-100.0% | 1700K(R255,G0, B0, L45) |
| 9CH | 0-255 | Color tint | 0 - no change 1-64 magenta shift from low to max, 65-127 magenta shift from max to smallest, 128 no shift, 129 -192 green shift from low to max, 193 - 254 green shift from max to smallest, 255 no shift. |
| 10CH | 0-255 | 0%-100% | Color Marcos |
|  | 0-45 | / | No Function |
|  | 46-50 | Cyan | R=000,G=255, B=255, L=000 |
|  | 51-55 | Yellow | R=255,G=255, B=000, L=000 |
|  | 56-60 | Magenta | R=255,G=000, B=255, L=000 |
|  | 61-65 | Medium Yellow | R= 156,G= 126,B=000,L=000 |
|  | 66-70 | Straw Tint | R= 152,G= 115,B=009,L=000 |
|  | 71-75 | Surprise Peach | R= 198,G= 114,B=009,L=000 |
|  | 76-80 | Fire | R=255,G=056, B=000, L=000 |
|  | 81-85 | Medium Amber | R=255,G= 135, B=000, L=000 |
|  | 86-90 | Gold Amber | R=255,G= 100, B=000, L=000 |
|  | 91-95 | Dark Amber | R=255,G=065, B=000, L=000 |
|  | 96-100 | Sunrise Red | R=255,G=083, B=002, L=000 |
|  | 101-105 | Light Pink | R=255,G= 112,B=000,L= 141 |
|  | 106-110 | Medium Pink | R=255,G=087,B=000,L= 107 |
|  | 111-115 | Pink Carnation | R=255,G= 107,B=000,L= 130 |
|  | 116-120 | Light Lavender | R=243,G= 117,B=039,L= 197 |
|  | 121-125 | Sky Blue | R=000,G=255, B= 135, L=000 |
|  | 126-130 | Just Blue | R=000,G= 194, B= 130,L=000 |
|  | 131-135 | Dark yellow green | R=041,G=219, B=000, L=000 |
|  | 136-140 | Spring Yellow | R=245,G=202, B=000, L=000 |
|  | 141-145 | Light Amber | R=237,G= 163, B=000, L=000 |
|  | 146-150 | Straw Tint | R=230,G= 160, B=000, L=069 |
|  | 151-155 | Deep Amber | R=255,G= 166, B=000, L=000 |
|  | 156-160 | Orange | R=255,G= 122, B=000, L=000 |
|  | 161-165 | Light Rose | R=255,G= 141, B=031, L=000 |
|  | 166-170 | Middle Rose | R=217,G= 130, B=028, L=000 |
|  | 171-175 | Dark Pink | R=255,G= 109, B=033, L=000 |
|  | 176-180 | Magenta | R=255,G=020, B=015, L=000 |
|  | 181-185 | Peacock Blue | R=051,G=255, B=051, L=000 |
|  | 186-190 | Steel Blue | R=206,G=255, B=056, L=000 |
|  | 191-195 | Light Blue | R=074,G=255, B=082, L=000 |
|  | 196-200 | Dark Blue | R=000,G= 186, B=255, L=000 |
|  | 201-205 | Leaf Green | R=206,G=255, B=000, L=000 |
|  | 206-210 | Dark Green | R=084,G=255, B=013, L=000 |
|  | 211-215 | Mauve | R=227,G=041, B=056, L=000 |
|  | 216-220 | Deep Golden Amber | R=255,G=058, B=000, L=000 |
|  | 221-225 | Pale Lavender | R=255,G= 197, B=061, L=000 |
|  | 226-230 | Primary Green | R=077,G=255, B=000, L=000 |
|  | 231-235 | Bright Blue | R=000,G=255, B=087, L=000 |
|  | 235-240 | Apricot | R=255,G= 143, B=013, L=000 |
|  | 241-245 | Pale Gold | R=253,G= 171, B=026, L=000 |
|  | 246-250 | Deep Orange | R=222,G=084, B=000, L=000 |
|  | 251-255 | Flame Red | R=255,G=046, B=002, L=000 |

20CH Mode

| CH | Value | Percent | Function |
| --- | --- | --- | --- |
| 1CH | 0-255 | 0%-100% | Background Dimmer |
| 2CH | 0-255 | 0%-100% | Background Dimmer Fine |
| 3CH | 0-255 | 0%-100% | Strobe of Marcos Effect |
|  | 0-5 | 0%-1.9% | No Function |
|  | 6-55 | 2.3%-21.5% | Synchronous strobe |
|  | 56-105 | 21.9%-41.1% | Pulse strobe |
|  | 106-155 | 41.5%-60.7% | Random strobe |
|  | 156-205 | 61.1%-80.3% | Fade Strobe1 |
|  | 206-255 | 80.7%-100% | Fade Strobe2 |
| 4CH | 0-255 | 0%-100% | Marcos Red |
| 5CH | 0-255 | 0%-100% | Marcos Green |
| 6CH | 0-255 | 0%-100% | Marcos Blue |
| 7CH | 0-255 | 0%-100% | Marcos Lime |
| 8CH | 0-255 | 0%-100% | Master Dimmer of Marcos Effect |
| 9CH | 0-255 | 0%-100% | Fine Dimmer of Marcos Effect |
| 10CH | 0-255 | 0%-100% | Strobe of Marcos Effect |
|  | 0-5 | 0%-1.9% | No Function |
|  | 6-55 | 2.3%-21.5% | Synchronous strobe |
|  | 56-105 | 21.9%-41.1% | Pulse strobe |
|  | 106-155 | 41.5%-60.7% | Random strobe |
|  | 156-205 | 61.1%-80.3% | Fade Strobe1 |
|  | 206-255 | 80.7%-100% | Fade Strobe2 |
| 11CH | 0-255 | 0%-100% | Marcos Red |
| 12CH | 0-255 | 0%-100% | Marcos Green |
| 13CH | 0-255 | 0%-100% | Marcos Blue |
| 14CH | 0-255 | 0%-100% | Marcos Lime |
| 15CH | 0-255 | 0%-100% | Program Effects |
|  | 00-5 | 0%-1.9% | No Function |
|  | 6-20 | 2.3%-7.8% | Program 1 |
|  | 21-35 | 8.2%-13.7% | Program 2 |
|  | 36-50 | 14.1%-19.6% | Program 3 |
|  | 51-65 | 20.0%-25.4% | Program 4 |
|  | 66-80 | 25.8%-31.3% | Program 5 |
|  | 81-95 | 31.7%-37.2% | Program 6 |
|  | 96-110 | 37.6%-43.1% | Program 7 |
|  | 111-125 | 43.5%-49.0% | Program 8 |
|  | 126-140 | 49.4%-54.9% | Program 9 |
|  | 141-155 | 55.2%-60.7% | Program 10 |
|  | 156-170 | 61.1%-66.6% | Program 11 |
|  | 171-185 | 67.0%-72.5% | Program 12 |
|  | 186-200 | 72.9%-78.4% | Program 13 |
|  | 201-215 | 78.8%-84.3% | Program 14 |
|  | 216-230 | 84.7%-90.1% | Program 15 |
|  | 231-245 | 90.5%-96.0% | Program 16 |
|  | 246-250 | 96.4%-98.0% | Program 17 |
|  | 251-255 | 98.4%-100% | Auto Program |
| 16CH | 0-255 | 0%-100% | Program Speed (Slow to Fast) |
| 17CH | 0-255 | 0%-100% | CTO Preset |
|  | 0-15 | 0%-5.8% | No Function |
|  | 16-30 | 6.2%-11.7% | 8000K（R255,G200, B155, L255) |
|  | 36-45 | 13.7%-20.0% | 7500K(R255,G174, B140,L255) |
|  | 46-60 | 14.1%-23.5% | 6500K(R255,G152, B124,L255) |
|  | 61-75 | 23.9%-29.4% | 6000K(R255,G136, B105,L255) |
|  | 76-90 | 29.8%-35.2% | 5600K（R255,G157, B158, L255) |
|  | 91-105 | 35.6%-41.1% | 5000K(R255,G124, B90, L255) |
|  | 106-120 | 41.5%-47.0% | 4500K(R255,G64, B63, L255) |
|  | 121-135 | 47.4%-52.9% | 4000K(R255,G32, B48, L255) |
|  | 136-150 | 53.3%-58.8% | 3500K(R255,G6, B33, L255) |
|  | 151-165 | 59.2%-64.7% | 3200K(R255,G0, B24, L255) |
|  | 166-180 | 65.0%-70.5% | 3000K(R255,G0, B20, L162) |
|  | 181-195 | 70.9%-76.4% | 2700K(R255,,G0,B13,L113) |
|  | 196-210 | 76.8%-82.3% | 2500K(R255,G0, B10, L95) |
|  | 211-225 | 82.7%-88.2% | 2300K(R255,G0, B7, L87) |
|  | 226-240 | 88.6%-94.1% | 2000K(R255,G0, B3, L68) |
|  | 241-255 | 94.5%-100.0% | 1700K(R255,G0, B0, L45) |
| 18CH | 0-255 | Color tint | 0 - no change 1-64 magenta shift from low to max, 65-127 magenta shift from max to smallest, 128 no shift, 129 -192 green shift from low to max, 193 - 254 green shift from max to smallest, 255 no shift. |
| 19CH | 0-255 | 0%-100% | Color Marcos |
|  | 0-45 | Macro Off | No Function |
|  | 46-50 | Cyan | R=000,G=255, B=255, L=000 |
|  | 51-55 | Yellow | R=255,G=255, B=000, L=000 |
|  | 56-60 | Magenta | R=255,G=000, B=255, L=000 |
|  | 61-65 | Medium Yellow | R= 156,G= 126,B=000,L=000 |
|  | 66-70 | Straw Tint | R= 152,G= 115,B=009,L=000 |
|  | 71-75 | Surprise Peach | R= 198,G= 114,B=009,L=000 |
|  | 76-80 | Fire | R=255,G=056, B=000, L=000 |
|  | 81-85 | Medium Amber | R=255,G= 135, B=000,L=000 |
|  | 86-90 | Gold Amber | R=255,G= 100, B=000,L=000 |
|  | 91-95 | Dark Amber | R=255,G=065, B=000, L=000 |
|  | 96-100 | Sunrise Red | R=255,G=083, B=002, L=000 |
|  | 101-105 | Light Pink | R=255,G= 112,B=000,L= 141 |
|  | 106-110 | Medium Pink | R=255,G=087,B=000,L= 107 |
|  | 111-115 | Pink Carnation | R=255,G= 107,B=000,L= 130 |
|  | 116-120 | Light Lavender | R=243,G= 117,B=039,L= 197 |
|  | 121-125 | Sky Blue | R=000,G=255,B= 135, L=000 |
|  | 126-130 | Just Blue | R=000,G= 194,B= 130,L=000 |
|  | 131-135 | Dark yellow green | R=041,G=219, B=000, L=000 |
|  | 136-140 | Spring Yellow | R=245,G=202, B=000, L=000 |
|  | 141-145 | Light Amber | R=237,G= 163, B=000,L=000 |
|  | 146-150 | Straw Tint | R=230,G= 160, B=000,L=069 |
|  | 151-155 | Deep Amber | R=255,G= 166, B=000,L=000 |
|  | 156-160 | Orange | R=255,G= 122, B=000,L=000 |
|  | 161-165 | Light Rose | R=255,G= 141, B=031,L=000 |
|  | 166-170 | Middle Rose | R=217,G= 130, B=028,L=000 |
|  | 171-175 | Dark Pink | R=255,G= 109, B=033,L=000 |
|  | 176-180 | Magenta | R=255,G=020, B=015, L=000 |
|  | 181-185 | Peacock Blue | R=051,G=255, B=051, L=000 |
|  | 186-190 | Steel Blue | R=206,G=255, B=056, L=000 |
|  | 191-195 | Light Blue | R=074,G=255, B=082, L=000 |
|  | 196-200 | Dark Blue | R=000,G= 186, B=255,L=000 |
|  | 201-205 | Leaf Green | R=206,G=255, B=000, L=000 |
|  | 206-210 | Dark Green | R=084,G=255, B=013, L=000 |
|  | 211-215 | Mauve | R=227,G=041, B=056, L=000 |
|  | 216-220 | Deep Golden Amber | R=255,G=058, B=000, L=000 |
|  | 221-225 | Pale Lavender | R=255,G= 197, B=061,L=000 |
|  | 226-230 | Primary Green | R=077,G=255, B=000, L=000 |
|  | 231-235 | Bright Blue | R=000,G=255, B=087, L=000 |
|  | 235-240 | Apricot | R=255,G= 143, B=013,L=000 |
|  | 241-245 | Pale Gold | R=253,G= 171, B=026,L=000 |
|  | 246-250 | Deep Orange | R=222,G=084, B=000, L=000 |
|  | 251-255 | Flame Red | R=255,G=046, B=002, L=000 |
| 20CH | 0-255 | 0%-100% | Dimmer Mode |
|  | 0-105 | 0%-41.1% | Dimmer Mode1 |
|  | 106-150 | 41.5%-58.8% | Dimmer Mode2 |
|  | 151-200 | 59.2%-78.4% | Dimmer Mode3 |
|  | 201-255 | 78.8%-100% | Dimmer Mode4 |

33CH Mode

| CH | Value | Percent | Function |
| --- | --- | --- | --- |
| 1CH | 0-255 | 0%-100% | Background Dimmer |
| 2CH | 0-255 | 0%-100% | Strobe Function of Background |
| 3CH | 0-255 | 0%-100% | Background Red |
| 4CH | 0-255 | 0%-100% | Background Green |
| 5CH | 0-255 | 0%-100% | Background Blue |
| 6CH | 0-255 | 0%-100% | Background Lime |
| 7CH | 0-255 | 0%-100% | Master Dimmer |
| 8CH | 0-255 | 0%-100% | Master Strobe |
| 9CH | 0-255 | 0%-100% | Foreground Red |
| 10CH | 0-255 | 0%-100% | Foreground Green |
| 11CH | 0-255 | 0%-100% | Foreground Blue |
| 12CH | 0-255 | 0%-100% | Foreground Lime |
| 13CH | 0-255 | 0%-100% | Program Effects |
|  | 00-5 | 0%-1.9% | No Function |
|  | 6-20 | 2.3%-7.8% | Program 1 |
|  | 21-35 | 8.2%-13.7% | Program 2 |
|  | 36-50 | 14.1%-19.6% | Program 3 |
|  | 51-65 | 20.0%-25.4% | Program 4 |
|  | 66-80 | 25.8%-31.3% | Program 5 |
|  | 81-95 | 31.7%-37.2% | Program 6 |
|  | 96-110 | 37.6%-43.1% | Program 7 |
|  | 111-125 | 43.5%-49.0% | Program 8 |
|  | 126-140 | 49.4%-54.9% | Program 9 |
|  | 141-155 | 55.2%-60.7% | Program 10 |
|  | 156-170 | 61.1%-66.6% | Program 11 |
|  | 171-185 | 67.0%-72.5% | Program 12 |
|  | 186-200 | 72.9%-78.4% | Program 13 |
|  | 201-215 | 78.8%-84.3% | Program 14 |
|  | 216-230 | 84.7%-90.1% | Program 15 |
|  | 231-245 | 90.5%-96.0% | Program 16 |
|  | 246-250 | 96.4%-98.0% | Program 17 |
|  | 251-255 | 98.4%-100% | Auto Program |
| 14CH | 0-255 | 0%-100% | Program Speed |
| 15CH | 0-255 | 0%-100% | LED 1 Dimmer Dimmer of One LED. Channels 9-12 defines the color |
| … | … | … | … |
| 32CH | 0-255 | 0%-100% | LED 18 Dimmer Dimmer of One LED. Channels 9-12 defines the color |
| 33ch | 0-255 | Mirror | Flips pixel order (0-Off, 1-255-On) |

37CH Mode

| CH | Value | Percent | Function |
| --- | --- | --- | --- |
| 1CH | 0-255 | 0%-100% | Background Dimmer |
| 2CH | 0-255 | 0%-100% | Background Strobe |
| 3CH | 0-255 | 0%-100% | Background Red |
| 4CH | 0-255 | 0%-100% | Background Green |
| 5CH | 0-255 | 0%-100% | Background Blue |
| 6CH | 0-255 | 0%-100% | Background Lime |
| 7CH | 0-255 | 0%-100% | Master Dimmer |
| 8CH | 0-255 | 0%-100% | Master Strobe |
| 9CH | 0-255 | 0%-100% | Foreground Red |
| 10CH | 0-255 | 0%-100% | Foreground Green |
| 11CH | 0-255 | 0%-100% | Foreground Blue |
| 12CH | 0-255 | 0%-100% | Foreground Lime |
| 13CH | 0-255 | 0%-100% | CTO Preset |
|  | 0-15 | 0%-5.8% | No Function |
|  | 16-30 | 6.2%-11.7% | 8000K（R255,G200, B155, L255) |
|  | 36-45 | 13.7%-20.0% | 7500K(R255,G174, B140,L255) |
|  | 46-60 | 14.1%-23.5% | 6500K(R255,G152, B124,L255) |
|  | 61-75 | 23.9%-29.4% | 6000K(R255,G136, B105,L255) |
|  | 76-90 | 29.8%-35.2% | 5600K（R255,G157, B158, L255) |
|  | 91-105 | 35.6%-41.1% | 5000K(R255,G124, B90, L255) |
|  | 106-120 | 41.5%-47.0% | 4500K(R255,G64, B63, L255) |
|  | 121-135 | 47.4%-52.9% | 4000K(R255,G32, B48, L255) |
|  | 136-150 | 53.3%-58.8% | 3500K(R255,G6, B33, L255) |
|  | 151-165 | 59.2%-64.7% | 3200K(R255,G0, B24, L255) |
|  | 166-180 | 65.0%-70.5% | 3000K(R255,G0, B20, L162) |
|  | 181-195 | 70.9%-76.4% | 2700K(R255,,G0,B13,L113) |
|  | 196-210 | 76.8%-82.3% | 2500K(R255,G0, B10, L95) |
|  | 211-225 | 82.7%-88.2% | 2300K(R255,G0, B7, L87) |
|  | 226-240 | 88.6%-94.1% | 2000K(R255,G0, B3, L68) |
|  | 241-255 | 94.5%-100.0% | 1700K(R255,G0, B0, L45) |
| 14CH | 0-255 | 0%-100% | Color Marcos |
|  | 0-45 | Macro Off | No Function |
|  | 46-50 | Cyan | R=000,G=255, B=255, L=000 |
|  | 51-55 | Yellow | R=255,G=255, B=000, L=000 |
|  | 56-60 | Magenta | R=255,G=000, B=255, L=000 |
|  | 61-65 | Medium Yellow | R= 156,G= 126,B=000,L=000 |
|  | 66-70 | Straw Tint | R= 152,G= 115,B=009,L=000 |
|  | 71-75 | Surprise Peach | R= 198,G= 114,B=009,L=000 |
|  | 76-80 | Fire | R=255,G=056, B=000, L=000 |
|  | 81-85 | Medium Amber | R=255,G= 135, B=000,L=000 |
|  | 86-90 | Gold Amber | R=255,G= 100, B=000,L=000 |
|  | 91-95 | Dark Amber | R=255,G=065, B=000, L=000 |
|  | 96-100 | Sunrise Red | R=255,G=083, B=002, L=000 |
|  | 101-105 | Light Pink | R=255,G= 112,B=000,L= 141 |
|  | 106-110 | Medium Pink | R=255,G=087,B=000,L= 107 |
|  | 111-115 | Pink Carnation | R=255,G= 107,B=000,L= 130 |
|  | 116-120 | Light Lavender | R=243,G= 117,B=039,L= 197 |
|  | 121-125 | Sky Blue | R=000,G=255,B= 135, L=000 |
|  | 126-130 | Just Blue | R=000,G= 194,B= 130,L=000 |
|  | 131-135 | Dark yellow green | R=041,G=219, B=000, L=000 |
|  | 136-140 | Spring Yellow | R=245,G=202, B=000, L=000 |
|  | 141-145 | Light Amber | R=237,G= 163, B=000,L=000 |
|  | 146-150 | Straw Tint | R=230,G= 160, B=000,L=069 |
|  | 151-155 | Deep Amber | R=255,G= 166, B=000,L=000 |
|  | 156-160 | Orange | R=255,G= 122, B=000,L=000 |
|  | 161-165 | Light Rose | R=255,G= 141, B=031,L=000 |
|  | 166-170 | Middle Rose | R=217,G= 130, B=028,L=000 |
|  | 171-175 | Dark Pink | R=255,G= 109, B=033,L=000 |
|  | 176-180 | Magenta | R=255,G=020, B=015, L=000 |
|  | 181-185 | Peacock Blue | R=051,G=255, B=051, L=000 |
|  | 186-190 | Steel Blue | R=206,G=255, B=056, L=000 |
|  | 191-195 | Light Blue | R=074,G=255, B=082, L=000 |
|  | 196-200 | Dark Blue | R=000,G= 186, B=255,L=000 |
|  | 201-205 | Leaf Green | R=206,G=255, B=000, L=000 |
|  | 206-210 | Dark Green | R=084,G=255, B=013, L=000 |
|  | 211-215 | Mauve | R=227,G=041, B=056, L=000 |
|  | 216-220 | Deep Golden Amber | R=255,G=058, B=000, L=000 |
|  | 221-225 | Pale Lavender | R=255,G= 197, B=061,L=000 |
|  | 226-230 | Primary Green | R=077,G=255, B=000, L=000 |
|  | 231-235 | Bright Blue | R=000,G=255, B=087, L=000 |
|  | 235-240 | Apricot | R=255,G= 143, B=013,L=000 |
|  | 241-245 | Pale Gold | R=253,G= 171, B=026,L=000 |
|  | 246-250 | Deep Orange | R=222,G=084, B=000, L=000 |
|  | 251-255 | Flame Red | R=255,G=046, B=002, L=000 |
| 15CH | 0-255 | 0%-100% | Foreground Macro |
|  | 00-5 | 0%-1.9% | No Function |
|  | 6-20 | 2.3%-7.8% | Program 1 |
|  | 21-35 | 8.2%-13.7% | Program 2 |
|  | 36-50 | 14.1%-19.6% | Program 3 |
|  | 51-65 | 20.0%-25.4% | Program 4 |
|  | 66-80 | 25.8%-31.3% | Program 5 |
|  | 81-95 | 31.7%-37.2% | Program 6 |
|  | 96-110 | 37.6%-43.1% | Program 7 |
|  | 111-125 | 43.5%-49.0% | Program 8 |
|  | 126-140 | 49.4%-54.9% | Program 9 |
|  | 141-155 | 55.2%-60.7% | Program 10 |
|  | 156-170 | 61.1%-66.6% | Program 11 |
|  | 171-185 | 67.0%-72.5% | Program 12 |
|  | 186-200 | 72.9%-78.4% | Program 13 |
|  | 201-215 | 78.8%-84.3% | Program 14 |
|  | 216-230 | 84.7%-90.1% | Program 15 |
|  | 231-245 | 90.5%-96.0% | Program 16 |
|  | 246-250 | 96.4%-98.0% | Program 17 |
|  | 251-255 | 96.4%-100% | Auto Run Program |
| 16CH | 0-255 | 0%-100% | Foreground Program Speed |
| 17CH | 0-255 | 0%-100% | Background Program Effects |
|  | 00-5 | 0%-1.9% | No Function |
|  | 6-20 | 2.3%-7.8% | Program 1 |
|  | 21-35 | 8.2%-13.7% | Program 2 |
|  | 36-50 | 14.1%-19.6% | Program 3 |
|  | 51-65 | 20.0%-25.4% | Program 4 |
|  | 66-80 | 25.8%-31.3% | Program 5 |
|  | 81-95 | 31.7%-37.2% | Program 6 |
|  | 96-110 | 37.6%-43.1% | Program 7 |
|  | 111-125 | 43.5%-49.0% | Program 8 |
|  | 126-140 | 49.4%-54.9% | Program 9 |
|  | 141-155 | 55.2%-60.7% | Program 10 |
|  | 156-170 | 61.1%-66.6% | Program 11 |
|  | 171-185 | 67.0%-72.5% | Program 12 |
|  | 186-200 | 72.9%-78.4% | Program 13 |
|  | 201-215 | 78.8%-84.3% | Program 14 |
|  | 216-230 | 84.7%-90.1% | Program 15 |
|  | 231-245 | 90.5%-96.0% | Program 16 |
|  | 246-250 | 96.4%-98.0% | Program 17 |
|  | 251-255 | 98.4%-100% | Auto Run Program |
| 18CH | 0-255 | 0%-100% | Background Program Speed |
| 19CH | 0-255 | 0%-100% | LED 1 Dimmer Dimmer of One LED. Channels 9-12 defines the color |
| … | … | … | … |
| 36CH | 0-255 | 0%-100% | LED 18 Dimmer |
| 37CH | 0-255 | Mirror | Flips pixel order (0-Off, 1-255-On) Dimmer of One LED. Channels 9-12 defines the color |

77CH Mode

| CH | Value | Percent | Function |
| --- | --- | --- | --- |
| 1CH | 0-255 | 0%-100% | Master Dimmer |
| 2CH | 0-255 | 0%-100% | Fine Dimmer |
| 3CH | 0-255 | 0%-100% | Strobe of Marcos Effect |
|  | 0-5 | 0%-1.9% | No Function |
|  | 6-55 | 2.3%-21.5% | Synchronous strobe |
|  | 56-105 | 21.9%-41.1% | Pulse strobe |
|  | 106-155 | 41.5%-60.7% | Random strobe |
|  | 156-205 | 61.1%-80.3% | Fade Strobe1 |
|  | 206-255 | 80.7%-100% | Fade Strobe2 |
| 4CH | 0-255 | 0%-100% | Color Temperature preset |
|  | 0-15 | 0%-5.8% | No Function |
|  | 16-30 | 6.2%-11.7% | 8000K（R255,G200, B155, L255) |
|  | 36-45 | 13.7%-20.0% | 7500K(R255,G174, B140,L255) |
|  | 46-60 | 14.1%-23.5% | 6500K(R255,G152, B124,L255) |
|  | 61-75 | 23.9%-29.4% | 6000K(R255,G136, B105,L255) |
|  | 76-90 | 29.8%-35.2% | 5600K（R255,G157, B158, L255) |
|  | 91-105 | 35.6%-41.1% | 5000K(R255,G124, B90, L255) |
|  | 106-120 | 41.5%-47.0% | 4500K(R255,G64, B63, L255) |
|  | 121-135 | 47.4%-52.9% | 4000K(R255,G32, B48, L255) |
|  | 136-150 | 53.3%-58.8% | 3500K(R255,G6, B33, L255) |
|  | 151-165 | 59.2%-64.7% | 3200K(R255,G0, B24, L255) |
|  | 166-180 | 65.0%-70.5% | 3000K(R255,G0, B20, L162) |
|  | 181-195 | 70.9%-76.4% | 2700K(R255,,G0,B13,L113) |
|  | 196-210 | 76.8%-82.3% | 2500K(R255,G0, B10, L95) |
|  | 211-225 | 82.7%-88.2% | 2300K(R255,G0, B7, L87) |
|  | 226-240 | 88.6%-94.1% | 2000K(R255,G0, B3, L68) |
|  | 241-255 | 94.5%-100.0% | 1700K(R255,G0, B0, L45) |
| 5CH | 0-255 | 0%-100% | Color Marcos |
|  | 0-45 |  | No Function |
|  | 46-50 | Cyan | R=000,G=255, B=255, L=000 |
|  | 51-55 | Yellow | R=255,G=255, B=000, L=000 |
|  | 56-60 | Magenta | R=255,G=000, B=255, L=000 |
|  | 61-65 | Medium Yellow | R= 156,G= 126,B=000,L=000 |
|  | 66-70 | Straw Tint | R= 152,G= 115,B=009,L=000 |
|  | 71-75 | Surprise Peach | R= 198,G= 114,B=009,L=000 |
|  | 76-80 | Fire | R=255,G=056, B=000, L=000 |
|  | 81-85 | Medium Amber | R=255,G= 135, B=000,L=000 |
|  | 86-90 | Gold Amber | R=255,G= 100, B=000,L=000 |
|  | 91-95 | Dark Amber | R=255,G=065, B=000, L=000 |
|  | 96-100 | Sunrise Red | R=255,G=083, B=002, L=000 |
|  | 101-105 | Light Pink | R=255,G= 112,B=000,L= 141 |
|  | 106-110 | Medium Pink | R=255,G=087,B=000,L= 107 |
|  | 111-115 | Pink Carnation | R=255,G= 107,B=000,L= 130 |
|  | 116-120 | Light Lavender | R=243,G= 117,B=039,L= 197 |
|  | 121-125 | Sky Blue | R=000,G=255,B= 135, L=000 |
|  | 126-130 | Just Blue | R=000,G= 194,B= 130,L=000 |
|  | 131-135 | Dark yellow green | R=041,G=219, B=000, L=000 |
|  | 136-140 | Spring Yellow | R=245,G=202, B=000, L=000 |
|  | 141-145 | Light Amber | R=237,G= 163, B=000,L=000 |
|  | 146-150 | Straw Tint | R=230,G= 160, B=000,L=069 |
|  | 151-155 | Deep Amber | R=255,G= 166, B=000,L=000 |
|  | 156-160 | Orange | R=255,G= 122, B=000,L=000 |
|  | 161-165 | Light Rose | R=255,G= 141, B=031,L=000 |
|  | 166-170 | Middle Rose | R=217,G= 130, B=028,L=000 |
|  | 171-175 | Dark Pink | R=255,G= 109, B=033,L=000 |
|  | 176-180 | Magenta | R=255,G=020, B=015, L=000 |
|  | 181-185 | Peacock Blue | R=051,G=255, B=051, L=000 |
|  | 186-190 | Steel Blue | R=206,G=255, B=056, L=000 |
|  | 191-195 | Light Blue | R=074,G=255, B=082, L=000 |
|  | 196-200 | Dark Blue | R=000,G= 186, B=255,L=000 |
|  | 201-205 | Leaf Green | R=206,G=255, B=000, L=000 |
|  | 206-210 | Dark Green | R=084,G=255, B=013, L=000 |
|  | 211-215 | Mauve | R=227,G=041, B=056, L=000 |
|  | 216-220 | Deep Golden Amber | R=255,G=058, B=000, L=000 |
|  | 221-225 | Pale Lavender | R=255,G= 197, B=061,L=000 |
|  | 226-230 | Primary Green | R=077,G=255, B=000, L=000 |
|  | 231-235 | Bright Blue | R=000,G=255, B=087, L=000 |
|  | 235-240 | Apricot | R=255,G= 143, B=013,L=000 |
|  | 241-245 | Pale Gold | R=253,G= 171, B=026,L=000 |
|  | 246-250 | Deep Orange | R=222,G=084, B=000, L=000 |
|  | 251-255 | Flame Red | R=255,G=046, B=002, L=000 |
| 6CH | 0-255 | 0%-100.0% | LED1 RED |
| 7CH | 0-255 | 0%-100.0% | LED1 GREEN |
| 8CH | 0-255 | 0%-100.0% | LED1 BLUE |
| 9CH | 0-255 | 0%-100.0% | LED1 LIME |
| ...... | .......... | ........... | .......................... |
| ...... | .......... | ........... | .......................... |
| ...... | .......... | ........... | .......................... |
| ...... | .......... | ........... | .......................... |
| 74CH | 0-255 | 0%-100.0% | LED18 RED |
| 75CH | 0-255 | 0%-100.0% | LED18 GREEN |
| 76CH | 0-255 | 0%-100.0% | LED18 BLUE |
| 77CH | 0-255 | 0%-100.0% | LED18 LIME |

SPECIFICATIONS

CONSTRUCTION/PHYSICAL

Dimensions: 1000*85*210MM

Weight； 10.5KG

Exterior Color: Black

Housing Material: Magnesium-aluminum die-casting +aluminum profile

DYNAMIC EFFECTS

Dimmer: Electronic Shutter/Strobe: Electronic

Strobe Rate: 0-25Hz

Standalone: Yes

Color Mixing: Yes

Color Temperature Presets: Yes

Dim Modes:4 kinds

White Balance: Yes

CONNECTIONS

Power Connection: Seetronic Powercon IP65

Power Input/Output: Seetronic Powercon IP65

CONTROL

Control Protocol: DMX/ArtNet/sACN/RDM

DMX Channels: 4/10/20/33/37/77CH

Build-in Effects: Yes

Software Update: Via RJ45 Cables

ELECTRICAL

Power Supplier:24V+12V at 1000W with PFC

Power Linking: 3units @ 100V;5 units @ 120V; 7 units @ 208V; 9 units @ 230V; 10 units @ 240V

OPTIC

Light Source: 18pcs*25W RGBL 4IN1 LEDs.

Color Temperature: 2700-8000K

Installed Optics: 6°/45°(can be changed)

DIMENSIONAL DRAWINGS