User Manual

RGB Laser Beam IP

SAFETY PRECAUTIONS

This fixture is a sophisticated piece of electronic equipment.

To guarantee a smooth operation,it is important to follow all instructions and guidelines in this manual. Professional is not responsible for injuries and/or damages resulting from the misuse of this fixture due to the disregard of the information printed in this manual.

Only qualified and/or certified personnel should perform installation of this fixture and only the original rigging parts included with this fixture should be used for installation.

Any modifications to the fixture and/ or the included mounting hardware will void the original manufacturer’s warranty and increase the risk of damage and/or personal injury.

PROTECTION CLASS 1 - FIXTURE MUST BE PROPERLY GROUNDED.

THERE ARE NO USER SERVICEABLE PARTS INSIDE THIS UNIT.DO NOT

ATTEMPT ANY REPAIRS YOURSELF,AS DOING SO WILL VOID

YOUR MANUFACTURER’S WARRANTY.DAMAGES RESULTING

FROM MODIFICATIONS TO THIS FIXTURE AND/OR THE DISREGARD OF

SAFETY INSTRUCTIONS AND GUIDELINES IN THIS MANUAL VOID

THE MANUFACTURER’S WARRANTY AND ARE NOT SUBJECT TO ANY WARRANTY CLAIMS AND/OR REPAIRS.

DO NOT PLUG FIXTURE INTO A DIMMER PACK!

NEVER OPEN THIS FIXTURE WHILE IN USE!

UNPLUG POWER BEFORE SERVICING FIXTURE!

NEVER TOUCH FIXTURE DURING OPERATION,

AS IT MAY BE HOT! KEEP FLAMMABLE MATERIALS AWAY FROM FIXTURE!

NEVER LOOK DIRECTLY INTO THE LIGHT SOURCE!

RETINA INJURY RISK - MAY INDUCE BLINDNESS!

SENSITIVE PERSONS MAY SUFFER AN EPILEPTIC SHOCK!

ENSURE ALL CONNECTIONS AND END CAPS ARE PROPERLY SEALED

WITH A DIELECTRIC GREASE (AVAILABLE AT MOST ELECTRICAL SUPPLIERS)TO  PREVENT WATER CORROSION AND/OR ELECTRICAL SHORT CIRCUIT.

MINIMUM DISTANCE TO OBJECTS/SURFACES

MUST BE 3.3FEET (1METER)

MAXIMUM TEMP OF EXTERNAL SURFACE 185° F (85°C)

MINIMUM DISTANCE OF FLAMMABLE MATERIALS FROM THE SURFACE IS 1.6  FEET (0 .5 METER)

SAFETY PRECAUTIONS

•  DO NOT TOUCH the fixture housing during operation.Turn OFF the power and allow approximately 15 minutes for the fixture to cool down before servicing.

•  DO NOT shake fixture, and avoid brute force when installing and/or operating  fixture.

•  DO NOT operate fixture if the power cord is frayed, crimped, damaged and/or if any of the power cord connectors are damaged and do not insert into the fixture securely with ease.

•  NEVER force a power cord connector into the fixture.If the power cord or any of its connectors are damaged, replace it immediately with a new one of the same power rating.

•  DO NOT block any air ventilation slots.

•  All fan and air inlets must remain clean and never blocked.

•  Allow approx.6” (15cm) between fixture and other devices or a wall for proper cooling.

•  Always disconnect fixture from main power source before performing any type

of service and/or cleaning procedure.

•  Only handle the power cord by the plug end.Never pull out the plug by tugging the wire portion of the cord.

•  During the initial operation of this fixture, a light smoke or smell may emit from the  interior of the fixture.This is a normal process and is caused by excess paint in the  interior of the casing burning off from the heat associated with the lamp and will decrease gradually over time.

•   Consistent operational breaks will ensure fixture will function properly for many years.

•   ONLY use the original packaging and materials to transport the fixture in for service.

MAINTENANCE GUIDELINES

DISCONNECT POWER BEFORE PERFORMING ANY MAINTENANCE!

CLEANING

Frequent cleaning is recommended to ensure proper function,optimized light output, and an extended life.The frequency of cleaning depends on the environment in which the fixture operates: damp,smoky or particularly dirty environments can cause greater accumulation of dirt on the fixture’s optics.Periodically clean the external lens surface with a soft cloth to avoid dirt/debris accumulation.NEVER use alcohol, solvents, or ammonia-based cleaners .

MAINTENANCE

Regular inspections are recommended to ensure proper function and extended life.There are no user serviceable parts inside this fixture.Please refer all other service issues to an authorized our service technician.Should you need any spare parts, please order genuine parts from your local dealer.

Please refer to the following points during routine inspections:

•    A detailed electric check by an approved electrical engineer every three months, to make sure the circuit contacts are in good condition and prevent overheating.

•   Be sure all screws and fasteners are securely tightened at all times.Loose screws may fall out during normal operation, resulting in damage or injury as larger parts could fall.

•    Check for any deformations on the housing, color lenses, rigging hardware,and rigging points (ceiling,suspension, trussing).Deformations in the housing could allow for dust to enter into the fixture.Damaged rigging points or unsecured rigging could cause the fixture to fall and seriously injure a person(s).

•   Electric power supply cables must not show any damage,material fatigue or sediments.NEVER remove the ground prong from the power cable.

OVERVIEW

- System Menu LCD Display   2.Power Input

- FUSE    4.Valve   5.Power Output

- DMX 3 PIN IN   7.DMX 3 PIN OUT   8.DMX 5 PIN IN

- 9.DMX 5 PIN OUT   10.RJ45 Out    11.RJ45 IN

SYSTEM MENU

| Main Menu | Sub Menu | Option/Value | Option/Value | Option/Value |
| --- | --- | --- | --- | --- |
| Address | 001~512 |  |  |  |
| Channel Mode | 019 CH |  |  |  |
| Setting | Control Mode | DMX |  |  |
|  |  | ARTNET |  |  |
|  |  | sACN |  |  |
|  | Fan Mode | Auto |  |  |
|  |  | Silent |  |  |
|  |  | Full |  |  |
|  | Calibration | Invert Pan | On/off |  |
|  |  | Invert Tilt | On/off |  |
|  |  | Motor Adjust | Pan | 0-127 |
|  |  |  | Tilt | 0-127 |
|  |  |  | Gobo | 0-127 |
|  |  |  | Frost Start | 0-127 |
|  |  |  | Frost End | 0-127 |
|  |  |  | Prism 1 Start | 0-127 |
|  |  |  | Prism 1 End | 0-127 |
|  |  |  | Prism 1 Rot | 0-127 |
|  |  |  | Prism 2 Start | 0-127 |
|  |  |  | Prism 2 End | 0-127 |
|  |  |  | Prism 2 Rot | 0-127 |
|  |  |  | Focus Start | 0-127 |
|  |  |  | Focus End | 0-127 |
|  | Language | Chinese |  |  |
|  |  | English |  |  |
|  | Invert Screen | Off |  |  |
|  |  | On |  |  |
|  | Signal Keep | NO |  |  |
|  |  | YES |  |  |
|  | Screen Lock | NO |  |  |
|  |  | YES |  |  |
|  | Load Default | NO |  |  |
|  |  | YES |  |  |
| Network | Ethernet | IP:002.000.000.001 |  |  |
|  |  | MS:255.000.000.000 |  |  |
|  | Universe | Net(0-127):000-127 |  |  |
|  |  | Sub_Uni:000-255 |  |  |
| Manual | Pan | 000-255 |  |  |
|  | Pan Fine | 000-255 |  |  |
|  | Tilt | 000-255 |  |  |
|  | Tilt Fine | 000-255 |  |  |
|  | Pan&Tilt Speed | 000-255 |  |  |
|  | Strobe | 000-255 |  |  |
|  | Dimmer | 000-255 |  |  |
|  | Laser Switch | 000-255 |  |  |
|  | Red | 000-255 |  |  |
|  | Green | 000-255 |  |  |
|  | Bule | 000-255 |  |  |
|  | Gobo | 000-255 |  |  |
|  | Frost | 000-255 |  |  |
|  | Prism 1 | 000-255 |  |  |
|  | Prism 1 Rot | 000-255 |  |  |
|  | Prism 2 | 000-255 |  |  |
|  | Prism 2 Rot | 000-255 |  |  |
|  | Focus | 000-255 |  |  |
|  | Reset | 000-255 |  |  |
| Reset | Reset Effect |  |  |  |
|  | Reset Scan |  |  |  |
|  | Reset All |  |  |  |

DMX CHART

| Channel | Value | Function | Description |
| --- | --- | --- | --- |
| 1 | 0-255 | Pan | 0-540° |
| 2 | 0-255 | Pan Fine | 0-2° |
| 3 | 0-255 | Tilt | 0-270° |
| 4 | 0-255 | Tilt Fine | 0-1° |
| 5 | 0-255 | Pan&Tilt Speed | Slow to Fast |
| 6 | 0-3 | Strobe | Off |
|  | 4-99 |  | Synchronous strobe |
|  | 100-149 |  | Pulse strobe |
|  | 150-199 |  | Strobe |
|  | 200-249 |  | Random strobe |
|  | 250-255 |  | On |
| 7 | 0-255 | Dimmer | 0-100% Dimmer |
| 8 | 0-127 | Laser Switch | Off |
|  | 128-255 |  | On |
| 9 | 0-255 | Red | 0-100% Dimmer |
| 10 | 0-255 | Green | 0-100% Dimmer |
| 11 | 0-255 | Blue | 0-100% Dimmer |
| 12 | 0-7 | Pattern | White light hole |
|  | 8-14 |  | Pattern 1 |
|  | 15-21 |  | Pattern 2 |
|  | 22-28 |  | Pattern 3 |
|  | 29-35 |  | Pattern 4 |
|  | 36-42 |  | Pattern 5 |
|  | 43-49 |  | Pattern 6 |
|  | 50-56 |  | Pattern 7 |
|  | 57-63 |  | Pattern 8 |
|  | 64-70 |  | Pattern 9 |
|  | 71-77 |  | Pattern 10 |
|  | 78-84 |  | Pattern 11 |
|  | 85-91 |  | Pattern 12 |
|  | 92-98 |  | Pattern 13 |
|  | 99-105 |  | Pattern 1 shakes from slow to fast |
|  | 106-112 |  | Pattern 2 shakes from slow to fast |
|  | 113-119 |  | Pattern 3 shakes from slow to fast |
|  | 120-126 |  | Pattern 4 shakes from slow to fast |
|  | 127-133 |  | Pattern 5 shakes from slow to fast |
|  | 134-140 |  | Pattern 6 shakes from slow to fast |
|  | 141-147 |  | Pattern 7 shakes from slow to fast |
|  | 148-154 |  | Pattern 8 shakes from slow to fast |
|  | 155-161 |  | Pattern 9 shakes from slow to fast |
|  | 162-168 |  | Pattern 10 shakes from slow to fast |
|  | 169-175 |  | Pattern 11 shakes from slow to fast |
|  | 176-182 |  | Pattern 12 shakes from slow to fast |
|  | 183-189 |  | Pattern 13 shakes from slow to fast |
|  | 190-221 |  | Counterclockwise flow from fast to slow |
|  | 222-223 |  | Stop |
|  | 224-255 |  | Clockwise flow from slow to fast |
| 13 | 0-127 | Atomization | Cut out |
|  | 128-255 |  | Cut in |
| 14 | 0-127 | Prism 1 | Remove Prism 1 |
|  | 128-255 |  | Insert Prism 1 |
| 15 | 0-127 | Prism 1 Rotate | 0-400° |
|  | 128-189 |  | Forward flow from fast to slow |
|  | 190-193 |  | Stop |
|  | 194-255 |  | Reverse flow from slow to fast |
| 16 | 0-127 | Prism 2 | Remove Prism 2 |
|  | 128-255 |  | Insert Prism 2 |
| 17 | 0-127 | Prism 2 Rotate | 0-400° |
|  | 128-189 |  | Forward flow from fast to slow |
|  | 190-193 |  | Stop |
|  | 194-255 |  | Reverse flow from slow to fast |
| 18 | 0-255 | Zoom | Far to close |
| 19 | 230-235 | Reset | Motor reset after more than 5 seconds |
|  | 240-245 |  | Pan and Tile motor reset after more than 5 seconds |
|  | 250-255 |  | All motors reset after more than 5 seconds |

DIMENSIONAL DRAWINGS

SPECIFICATIONS

◆Luminous Flux Typical Value*(lm): ≥10000

◆Red Laser Wavelength Typical Value (nm): 638

◆Green Laser Wavelength Typical Value (nm): 520

◆Blue Laser Wavelength Typical Value (nm): 450

◆Initial Beam Diameter (mm): 80~120

◆Divergence Angle: 0.35°

◆Power of RGB Laser Source (W): 60W

◆System Power Consumption: 500W

◆Light Source Lifespan: ≥20000H

◆Beam Angle: Vertical 0-270°, Horizontal 0-540°

◆Control Precision: 16 bits

◆Control Protocol: DMX, Art-Net, sACN

◆Weight: 22KG

◆IP Rated:IP65

◆Operating Temperature: -20°C to 45°C